Anti-UAV Defense System

SYSTEM OVERVIEW

The anti-UAV defense system adopts an integrated architecture, comprising a detection subsystem, a countermeasure subsystem, and command control software. The detection subsystem employs multi-sensor technologies (radar, electro-optical devices, etc.) to achieve all-weather, omnidirectional detection, search, and tracking of low-altitude UAVs, automatically acquiring target parameters (range, azimuth, altitude, velocity, etc.). The countermeasure subsystem integrates soft/hard kill means (electronic jamming, weapon stations, DID, etc.) to neutralize threats. Command control software visualizes comprehensive situational awareness, assists in formulating defense strategies, and automatically guides directional jamming devices, weapon stations and DID to repel or intercept the targets.

It can be widely applied to the protection of key core targets such as position command posts, oil depots, ammunition depots, government security sites, airports, and energy infrastructure.

TECHNICAL SPECIFICATIONS

| Specifications |  | Parameters |
| --- | --- | --- |
| Radar | Band | X |
|  | Operation Distance | >14km(RCS=0.01㎡,DJI Phantom 4 Drone |
|  | Coverage Range | Single array: azimuth ±45° Quadruple arrays: azimuth 360° |
|  | Tracking Target | >1000 |
| Infrared Thermal Imaging Equipment | Detector | Uncooled VOx IR FPA |
|  | Detection Range | 3km |
|  | Resolution | 640×512 |
|  | Lens Focal Length | 38～190mmContinuous Zoom Lens |
| High-Definition Visible Light Equipment | Sensor | 1/1.8" Target Starlight-Level CMOS with ICR Color-to-Black Function |
|  | Detection Range | 3km |
|  | Resolution | 1920×1080 |
|  | Focal Length | 11～860mm High-Definition Motorized Zoom Lens |
| Directional Jamming Equipment | Effective Jamming Distance | ≥3km |
|  | Jamming Coverage Area | Azimuth∶0°～360°; Pitch∶30° ～90° |
|  | Jamming Frequency Band | 900M/1.6G/2.4G/5.8G |
| Weapon Station | Range | 200m（W-25） 800m（W-150） |
|  | Hit Probability | The hit rate of 1 hit in 3 attempts is ≥ 90% (W-25). The knockdown rate within 10 seconds is ≥ 60% (W-150). The knockdown rate within 30 seconds is ≥ 90% (W-150). |
| DID |  | Maximum Speed: 40m/s Range: 5km Interception Mode: AI autonomous interception |
| Dimensions |  | Detection section: 100×100×170 cm (L×W×H) Countermeasure section: 1170×970×900 cm (L×Wh×H) |
| Weight |  | Detection section: 240 kg Countermeasure section: 20 kg / 250 kg |
| Power Supply |  | AC 220V |