WEAPON STATION Sniper W-150

PRODUCT OVERVIEW

K-150 is a multi-platform, multi-role remote weapon station supporting multiple large-caliber weapons. It integrates automated detection, recognition, tracking, and precision strike capabilities against ground, surface, and low-speed aerial targets. Programmable ammunition enables effective engagement of concealed positions and LSS UAV targets.

- Multi-weapon reconfiguration

12.7mm Machine Gun

14.5mm Machine Gun

40mm Grenade Launcher

- Multi-platform deployment

Manned/Unmanned Surface Vessels

Light High-Mobility Vehicles

Light/Medium Unmanned Ground Chassis

Medium/Heavy Manned Armored Vehicles

Transport Vehicles, Self-Propelled Artillery.

Application

TECHNICAL SPECIFICATIONS

| Parameter | Specification | Notes |
| --- | --- | --- |
| Fit Guns and Ammunitions | 40mm grenade launcher Airburst ammunition |  |
| Ammunition Capacity | 30～60 rounds | Depend on guns |
| Weight | ≤150kg | Without guns and ammunitions |
| Elevation | -20° to +45° | Depend on guns |
| Azimuth | N×360˚ |  |
| Elevation slew rate | Max ≥45°/s, Min ≤0.03°/s |  |
| Azimuth slew rate | Max ≥60°/s, Min ≤0.03°/s |  |
| Maximum tracking rates | Azimuth ：Max ≥15°/s Elevation：Max ≥15°/s |  |
| Power input | 26±4VDC |  |
| Power Consumption | 2000W max |  |
| Operating Temperature | -20℃～+55℃ |  |
| EO | 1920×1080 Pixels 30FPS WFOV: 61.9˚×37.4˚ NFOV: 2.3˚×1.3˚ |  |
| IR | 640×512 Pixels 30FPS WFOV: 9˚×7˚ |  |
| tracking distance | # EO: drone target ≥1000m # IR: drone target ≥350m | RCS=0.01cm2 |
| Lidar ranging | # EO: drone target ≥800m # IR: drone target ≥350m | RCS=0.01cm2 |
| Counter-UAS Capability | ≥50% at 250m # Effective anti-UAVs: ～500m | DJI Phantom Depends on the accuracy of the weapon |