WEAPON STATION Sniper W-25

PRODUCT OVERVIEW

K-25 is a portable, ultralight remote weapon station designed for single-soldier carry and rapid random or fixed emplacement.It is primarily used by infantry squads to execute anti drone operations, infiltration, ambush, sniping, and guard control missions in urban, mountainous, and key-site combat environments, day or night.

It can be fixed and deployed using a tripod, and installed on fixed fortifications such as rooftops, windowsills, and walls; simultaneously, it can be mounted on light/small manned and unmanned vehicles. A dual-axis stabilization system is optional.

It can effectively neutralize or deter invading personnel targets and LSS UAV targets.

Equipped with a lightweight multi-sensor smart camera, it enables day/night reconnoiter through its color camera and infrared camera. The laser rangefinder provides precise target distance measurement.

Integrated with an intelligent fire control system. It enables automatic target acquisition, tracking, and aiming, thereby facilitating rapid, efficient, and precise target engagement. Simultaneously, it possesses intelligent capabilities including autonomous decision-making, continuous multi-target engagement, and coordination within multi-node, multi-asset combat networks..

Application：

TECHNICAL SPECIFICATIONS

| Parameter | Specification | Notes |
| --- | --- | --- |
| Fit Guns and Ammunitions | 5.56mm NATO rifle 7.62*39mm rifle |  |
| Ammunition Capacity | 30 rounds | Depend on guns |
| Weight | ≤25kg | Without guns and ammunitions |
| Elevation | -20° to +45° | Depend on guns |
| Azimuth | N×360˚ |  |
| Elevation slew rate | Max ≥45°/s, Min ≤0.03°/s |  |
| Azimuth slew rate | Max ≥60°/s, Min ≤0.03°/s |  |
| Maximum tracking rates | Azimuth ：Max ≥15°/s Elevation：Max ≥15°/s |  |
| Power input | 30～52VDC |  |
| Power Consumption | 200W max |  |
| Operating Temperature | -20℃～+55℃ |  |
| EO | 1920×1080 Pixels 30FPS WFOV: 25˚×14˚ NFOV: 7.5˚×4.2˚ |  |
| IR | 640×512 Pixels 30FPS WFOV: 9˚×7˚ |  |
| tracking distance | # EO: drone target ≥480m # IR: drone target ≥350m | RCS=0.01cm2 |
| Lidar ranging | # EO: drone target ≥480m # IR: drone target ≥350m | RCS=0.01cm2 |
| Counter-UAS Capability | ≥41.52% at 100m # Effective anti-UAVs: ～200m | DJI Phantom Depends on the accuracy of the weapon |